OrCAD Capture (CIS) to Cadence Allegro Interface

### Introduction

OrCAD now offers a new way to interface your OrCAD Capture and OrCAD Capture CIS 9.1 to Cadence Allegro PCB Layout (Unix and NT).  This new interface implements the functions of netlist creation, Device file creation and back-annotation file translation.

OrCAD’s new interface between Capture or Capture CIS and Allegro is implemented as a DLL and is accessed from the Accessories menu.  The DLL, ALLEGROS.DLL, is located in the Vendor sub-directory of the directory in which the Capture .EXE is located.

Netlist creation runs in a single step.  This produces an Allegro-formatted Netlist file, plus all necessary Device files from a Capture or Capture CIS schematic design.  The Netlist file and the Device files constitute the set of files that should be transferred to the Cadence Allegro system.

Back-annotation is a two-step process. The first step translates the Allegro (.BAF) file into a Capture Back-Annotation (.SWP) file.  This step also requires input from the original Capture-created Netlist, Unused section file and Device files. The second step uses the swap file as input to the Capture Back Annotate Tool function.

As we receive continued feedback on how to further improve this interface, new revisions are available for free download from the OrCAD Design Network located at http://www.orcad.com/odn/ under the Knowledge Base entries.

### Installation

Copy the ALLEGROS.DLL to the VENDOR subdirectory of the directory in which CAPTURE.EXE is installed (by default CAPTURE.EXE is installed in \PROGRAM FILES\ORCAD\CAPTURE).

### Cadence Allegro considerations

It is recommended that you read the Allegro documentation, supplied by Cadence, before attempting to prepare your OrCAD schematic design for netlisting to Allegro.  The most relevant sections concern Third Party Netlists and Device files.

Warning:  If you are planning to use the Back-Annotation translation function later, then the Netlist and Device files created by Capture must not be manually modified on the Allegro side, so take care to set up device and net properties completely before creating an Allegro compatible netlist.

Warning: Property names are case sensitive.  All Allegro Property names attached to Parts and Nets in Capture or Capture CIS should be in capital letters.

Warning: This tool does not support multi-section heterogeneous parts. These will be translated as single-section.

Part Properties in General: In order for part properties to produce the correct field values in the Netlist and Device files, part properties should be put on library parts, rather than on part instances in the schematic. To do this, select the part to which the properties are to be added and use the right-mouse button to activate the Part Editor. Use Options / Part Properties to add the properties listed below under sections titled PCB Footprint, Class and Additional Properties. After modifying the part, and closing the Part Editor window, use the Update All (part instances in the design) to make sure the properties are propagated to every like part in the design.

Pin Swaps: Pin Swapping in Allegro is enabled by PINSWAP specifications in the Device Files. PINSWAP specifications will be produced only if the Swap ID properties are set correctly on pin-swappable parts. To do this, select a section of the part of interest. Select Edit Part, then View/Package, then Edit/Properties. In the Properties spreadsheet set the Group value to 1 for each swappable pin in one section of the part. For example, on a 7400, set the Group to 1 for pins 1 and 2. Leave the other Group values blank. When you select a pin and edit its properties, this value will show as the “Swap Id” property. The default value is -1, which shows as blank on the property edit spreadsheet. You need only set the Swap Id property on the pins for any one section of the part.

Device Types: This tool generates Device files using the part DEVICE property, if present, or the part Value property. In order to minimize the number of Device files generated, parts should be grouped into like device categories using a common DEVICE property. Parts that have the same layout footprint and the same pin functionality are like devices. For example, all resistors of a given wattage, say 1/8, should have the same DEVICE property, say R1_8. Each unique value will generate a separate line in the $PACKAGES and $FUNCTIONS sections of the netlist, but only one Device file will be generated. If the resistors in a design don’t have common DEVICE properties, a Device file would be generated for each resistor value. This could result in the creation of numerous DEVICE files, which have to be managed and stored.

Since the DEVICE property is used to generate Device File names by appending “.txt” to the DEVICE property value, these must be valid file names, i.e., they may not contain certain characters such as “.” (period) or “\” or “/” (forward or backward slash)  “:” (colon) or space. It is best to use DEVICE property values that contain only letters, numbers, and the special characters “-“ and “_” (minus and underscore). An 8-character limit on DEVICE property values may be needed if the Device files are transported via a system that has the 8.3 format of DOS file names.

For further information on Device files please see the Appendix.

PCB Footprint: In order to generate the correct PackageName field in the netlist, and the correct PACKAGE value in Device files, each part must have valid a PCB Footprint property.

Class: In order to generate the correct Class field in Device Files, assign the correct CLASS property to each part.

Additional Properties: Part properties with the following names will be passed through to the netlist as “$A_PROPERTIES” at the end of the “$PACKAGES” section:

AUTO_RENAME

COMPONENT_WEIGHT

FIX_ALL

FIXED

HARD_LOCATION

INSERTION_CODE

NO_MOVE

NO_PIN_ESCAPE

NO_SWAP_COMP

NO_SWAP_GATE

NO_SWAP_GATE_EXT

NO_SWAP_PIN

PARTNUMBER

PIN_ESCAPE

PINUSE

PLACE_TAG

ROOM

TERMINATOR_PACK

The following part properties will generate PACKAGEPROP lines in the Device Files:

ALT_SYMBOLS

DEVICE_LABEL

HEIGHT

PART_NUMBER

REF_DES_FOR_ASSIGN

TOL

VALUE

No-Connect pins: In order to produce the correct pin count and the NC line in the Device File, no-connect pins must be treated as follows: Attach a property of the name “NC” to the part, and assign a property value of the form “n1,n2,n3, …” where “n1”, “n2”, etc. are the numbers of the no-connect pins. This can be done by selecting an instance of the part, selecting Edit / Part, then Options / Part Properties and adding a new user property named “NC” with the appropriate value. Upon closing the Edit Part window, select the Update All button so that all instances of this part receive the NC property.

Here’s an example of the use of the NC property: The 741 op-amp, in both metal and plastic packages, has 8 pins, but only the 7 pins that are actually used are accounted for in the Capture library part. In order to produce the correct pin count (8) in the Device file, an NC property whose value is “8” should be added to all 741’s placed in a design. (See “Part Properties in General” above for the suggested procedure to add the property).

To account for unconnected pins of multi-section parts, such as mounting holes of multi-row connectors, do not make the part heterogeneous with the mounting holes as pins on one section or distributed among the sections. Instead, make the part homogeneous and add a NC property to each section of the part, with the same pins listed for the NC property on all sections. In the Part Editor, use the View / Next Part / Previous Part menu picks to access all sections of the part, and place the same NC property on all sections.

Heterogeneous parts: Multi-section, heterogeneous parts are treated as single-section parts by this tool.

Net Properties: Net properties with the following names will be passed through to the netlist as “$A_PROPERTIES” at the end of the “$NETS” section:

DELAY_RULE

DIFFERENTIAL_PAIR

ECL

MAX_BOND_LENGTH

MIN_BOND_LENGTH

MIN_LINE_WIDTH

MIN_NECK_WIDTH

NET_PHYSICAL_TYPE

NET_SPACING_TYPE

NO_GLOSS

NO_RAT

NO_RIPUP

NO_ROUTE

NO_TEST

PROBE_NUMBER

ROUTE_PRIORITY

ROUTE_TO_SHAPE

SAME_NET

STUB_LENGTH

TS_ALLOWED

VIA_LIST

WEIGHT

### Netlist Generation

The Project Manager must be the active window. From the Accessories menu choose Allegro Netlist or Allegro Netlist (to Unix). If the Unix option is selected the Netlist file and Device files will be created without carriage returns, i.e. Unix-style with new-lines (linefeeds) only, and Device file names will be lower-case, per Unix standards, regardless of the case of the Capture DEVICE property value.

If the design has been modified since being saved, a dialog box will pop up stating that the design will be saved prior to netlisting. Select the OK button to save the file and proceed. If you select Cancel, the design will not be saved and no netlist will be generated.

Next, a dialog box will prompt for the name of the output netlist file. Device files will be created for each relevant device in the same directory as the Netlist file is created.

Errors will produce a warning or error dialog. Check the Session Log for details. It would be a good idea to check the Session Log every time this tool is run.

The outputs from Netlist generation are as follows:

- The Netlist File, of the name and path entered into the dialog. If this name is the same as an existing file, a prompt will appear asking if the existing file is to be overwritten.

- Device Files, created in the same directory as the netlist file. If Device Files of the same names already exist in the directory, they will be overwritten – no prompt warning the user of this will be displayed.

- An Unused Section file, created in the same directory as the netlist file, and with the same name as the netlist file, but with the extension “UNU”. For example, if the netlist file is “DESIGN1.NET”, the unused section file will be named “DESIGN1.UNU”. This file will overwrite an existing file of the same name with no warning.

The Netlist File and Device Files are used should be transferred to the Cadence Allegro system. The Unused Section file should be retained for use by back-annotation.

If the “to Unix” version was selected, the Netlist file and Device files will be created without carriage returns, i.e., Unix-style with new-lines (linefeeds) only. These files can be transferred to a Unix target without further translation.

### Back-Annotation Procedure

This function translates an Allegro produced Back-Annotation (.BAF) file into a file (.SWP) which is then input to Capture’s Back-Annotation function.

Warning: A valid Capture back-annotation file can be created only if the original Capture-created netlist and device files are used for the board design without modification.

The Project Manager must be the active window (it doesn’t matter what’s selected in the PM window). From the Accessories menu select Allegro Back Annotation. A dialog box will prompt for the name of the (input) Allegro-produced back-annotation (.BAF) file. Next, a dialog box will prompt for the name of the (input) Capture-produced Allegro Netlist File. Finally, a dialog box will prompt for the name of the (output) Capture Swap file. The Allegro Device Files and the Unused Section file must be available in the same directory as the input Netlist File (which is where the Netlisting process would have placed them automatically). As the translation runs, messages are sent to the Session Log. Check the log after running the translation.

When the translation is complete, a message box will pop up prompting you to run Gate and Pin Swap. Select the Design in the PM window, and select Tools/Back Annotation. Specify the generated Swap file name.

There is no Unix/non-Unix option in back-annotation, since the back-annotation translation can read a back-annotation file created on a Unix system without further translation.

### Appendix: Device Files

Device files provide Cadence Allegro with information about electrical and mechanical properties of parts. Device files are created automatically by the Capture Allegro Netlister, provided the correct properties are set up on parts and pins.

We recommend that parts of the same electrical function with the same footprint should share a device file. For example, all resistors of a given size, say ¼ watt, should use the same device file, that is, should have the same DEVICE property, say, “RES1_4”, even though their values may differ.

Here’s a sample Device File:

(Device file for 7400)

PACKAGE DIP14			Package name (PCB Footprint)

CLASS IC				Placement class

PINCOUNT 14				Total number of pins in device

PINORDER 7400 I0 I1 O		Defines pin names for a section

PINUSE 7400 IN IN OUT		Specifies pin functions

(relates to PINORDER)

PINSWAP 7400 I0 I1		Specifies swappable pins

FUNCTION G1 7400 1 2 3		Defines a section (relates to PINORDER)

FUNCTION G2 7400 4 5 6		Ditto

FUNCTION G3 7400 9 10 8		Ditto

FUNCTION G4 7400 12 13 11	Ditto

POWER VCC ; 14			Specifies power pins

GROUND GND ; 7			Ditto

END

This Device File defines the 7400 as having four sections (the four FUNCTION lines), each of which can be swapped with another in the same package, or with another equivalent function in a different package. The pins of each section are named “I0”, “I1” and “O” (the PINORDER line). The pin types are “IN” (I0) “IN” (I1) and “OUT” (O).  Pin types are used during routing for determining driving and terminating pins of a net, and by other tools (test generation and signal integrity). The “I1” and “I0” pins can be swapped within a section (the PINSWAP line).

Your goal should be to minimize the number of Device files, that is, your design should have the minimum number of unique DEVICE properties on parts, consistent with the electrical specifications of the design, and the need to avoid confusion to others working with the design. Ordinarily, each different IC type would have its own unique DEVICE value (hence, unique device file) but this needn’t be the case. For example, 7400 and 7402 DIPs have the same footprint (14-pin), the same power pins, and the same input and output pins for each section. They could have the same DEVICE property and, hence, share a Device file. However, the two parts would have different Values, so Allegro would not try to interchange them. If, for example the DEVICE type were “740X” for both, the netlist might appear as follows:

DIP14 ! 740X ! 7400 ; U1 U2

DIP14 ! 740X ! 7402 ; U3 U4

U1 and U2 have different values from U3 and U4. Although Allegro could interchange sections of U1 with sections of U2, and sections of U3 with sections of U4, it will not interchange sections of U1 or U2 with sections of U3 or U4.

In order to avoid confusing others who may have to work with the design, in this case it’s probably best to use separate DEVICE properties for the 7400 and 7402.

### OrCAD Contact Information

For North America

Phone: 800-671-9505
To place an order or receive the latest product, upgrade, extended support or training information, call between 6:00 a.m. and 5:00 p.m. Pacific Time. Or, send your name and phone number to info@orcad.com.

Technical Support Hotline 503- 671-9400 
The OrCAD Technical Support Hotline is open weekdays. For technical support of OrCAD Capture, OrCAD Express, and OrCAD Layout products, please call 503-671-9400, 6:00 a.m. to 11:00 a.m. or noon to 4:00 p.m. PST. For support by email, write to techsupport@orcad.com. Please include a valid product registration number in the Subject field of the email header.

The hotline is for registered users only, please have your registration number ready when you call.

Corporate Headquarters 
OrCAD, Inc. 
9300 S.W. Nimbus Avenue 
Beaverton, Oregon 97008
Phone: 503-671-9500
Fax: 503-671-9501 
E-mail: info@orcad.com 
Web site: http://www.orcad.com

For International OrCAD Support
Please contact your local OrCAD technical reseller.    OrCAD reseller contact information is available at http://www.orcad.com/company/company_f.htm

Revisions to this Document:

9/22/97 - added DELAY_RULE net property.

10/14/97 - added “:” (colon) as disallowed character in Device type

11/11/97 - Unix option and part property changes

11/25/97 - Added No-Connect pin handling

12/1/97 - Updated Net properties list, sorted Net and Part properties lists

1/13/98 - Changed handling of No-Connect Pins

4/8/98 - Clarified requirements for Device CLASS property

4/14/98 – Updated part property discussion

5/11/98 – Updated back-annotation information

5/14/98 – Document review

7/29/98 – Device Files Appendix added.

9/24/98 – Minor modifications incorporated to text.

3/28/99 – Updated to reflect 9.1-OrCAD product line.