Guide to User-Created Netlist Formatters

OrCAD now provides a means for users to generate their own netlist formatters using Microsoft Visual Basic. This is provided for flat netlisters only. The only hierarchical netlisters provided are those that are built into Capture - there is no facility for user-created hierarchical formatters at this time.

A user-created formatter consists of several pieces:

1) A Visual Basic form which defines the user interface, which consists of a single dialog in which the user specifies the output netlist file(s) and options (if any). A model is supplied by OrCAD but may be modified by the user. If the user’s formatter is called “MYFORMAT” then the suggested name for this module is “MYFORMAT.FRM”;

2) USERNETL.DLL, a DLL (dynamic link library), supplied by OrCAD. This DLL is called by the user’s Visual Basic code. It loads an intermediate netlist format (ONL - OrCAD Netlist Format, as produced by Capture Version 7.20 and later) file into internal data structures for access by the user’s netlist formatter code;

3) A declaration file, DLLDECL.BAS, in Visual Basic source, supplied by OrCAD, that defines the interface between the user’s formatter code and USERNETL.DLL;

4). The user’s netlist formatter code in Visual Basic source. Examples which can be used as a starting point accompany this tool-kit. If the user’s formatter is called “MYFORMAT” then the suggested name for this module is “MYFORMAT.BAS”.

The resulting formatter can be compiled into native code by Version 5.0 of Visual Basic, or compiled into p-code or interpreted by earlier versions of Visual Basic, including the Learning Edition. When Capture is to generate a netlist, it will launch the user’s formatter, including the Visual Basic interpreter, if necessary. Formatters can be written and debugged without Capture running, in fact, they can be developed on a machine without Capture installed. Likewise, the machine running the netlister under Capture need not have the Visual Basic development tools installed, only the appropriate Visual Basic run-time components.

Sample Formatters:

The best way to create your own new formatter is to start with one of the two sample Visual Basic Projects that are included in the USERNETL directory: WIRELIST.VBP and PCBIIL.VBP. These illustrate the two basic types of netlists: “Net-oriented” and “Part-oriented”. In the former, which is more common, the netlist has a separate Parts section (sometimes a separate file) and a Nets section. The Parts section lists all the parts, sometimes grouped by PCB Footprint or other common characteristic. The Net section lists the nets with all pins for a given net together. WIRELIST is an example of a net-oriented format.

PCBIIL is an example of a “Part-oriented” netlist. This type of netlist has only a single section listing all parts, and with each part, all of its pins and the net each pin is connected to. In this netlist type, the net (connectivity) information is spread among multiple parts, so visually checking nets for connectivity is difficult. On the other hand, since all the pins for a part are grouped together, it is easier to visually check the netlist to see that all known used pins for a given part are referenced in the netlist.

Debugging Your Netlist Formatter:

You don’t need to have Capture running to debug a VB netlist formatter, but you will need an input file, in OrCAD Netlist (.ONL) format. A small test file, TESTPG1.ONL, is supplied in the USERNETL directory. If you want to create test data from one of your own designs, load the design into Capture, create a netlist using any of the “Other” netlist formatters, and then exit from Capture. A .ONL file for your design will be found in your design directory. Copy this file to the USERNETL directory, or to the directory you are developing your formatter in.. In Visual Basic, under Project Properties / Make put the name of the .ONL file (without the .ONL extension) in the Command Line Arguments. The sample projects as supplied have “TESTPG1” in this field. Alternatively, you could leave the .ONL file in your design directory and set the Command Line Arguments field to point to the correct file and path.

There are two common errors that might occur as you are debugging your formatter. In DLLDECL.BAS, no explicit path is specified for USERNETL.DLL. Therefore, the most convenient location for it is the same directory in which the VB project is running. If the DLL is not found, the message box for “Run-Time error 53, File not found,” will be displayed. If you are developing your formatter in the USERNETL subdirectory of the CAPTURE directory, USERNETL.DLL will already have been placed there by the Capture Install procedure. If you are doing formatter development in another directory, then copy USERNETL.DLL to your working directory.

The second common error is the message “[FMT0016] Internal error – intermediate file not found”. USERNETL.DLL can’t find the .ONL file. Check the Command Line Arguments field as described in the first paragraph above for the correct file name and path. Do not include the .ONL extension.

Once you are satisfied your formatter is working correctly, to prepare it to run under Capture, compile the formatter to an EXE file, and copy that EXE to the NETFORMS subdirectory of the Capture directory. To invoke the formatter under Capture, select the Other tab of the Netlist dialog. You will see your formatter listed as a .EXE along with the other formatter DLL’s. Select your formatter. Enter a file name for Netlist File 1 - this is a dummy since it is not used. Then select OK. The dialog box for the formatter should display. The appropriate Visual Basic run-time DLL’s  must be accessible via your current PATH setting.

Note that if you debug a formatter outside the Capture environment, any messages that would be sent to Capture’s Session Log will instead cause message boxes to pop up.

To Add Options:

If you need user-selectable options, add check boxes or radio buttons to the main dialog. For each option, define (Dim) a Global Boolean variable in the formatter .BAS module. The Wirelist example includes check-box options. The option variable values are then checked in the netlist formatter code at the point at which the SwitchIsSet() functions would have been called in a formatter code in ‘c’. Specifically, in the Form module, implement the code for the Check Box as follows:

In the formatter-specific .BAS:

Global sw_x As Boolean

In the formatter-specific .FRM in the MakeNetlist_Click() function copy sw_x from Check1.Value (assuming the associated check-box’s name is Check1):

Private Sub MakeNetlist_Click()

sw_x = Check1.Value   ‘ ß added

filenameout = Text1.Text

Unload Me

If filenameout <> "" Then

Unload Me

Call Main(filenameout, vbNullString)

End

End If

End Sub

To add a Second Output File:

After the line Dim filenameout As String, add a second filename definition, such as:

Dim filename2 As String

To the main dialog, add a Text Box and Browse Command Button (plus a label for the text box for the second file name. Copy the body of BrowseButton_Click() into the body of the Click subroutine for the new browse button, except that the two lines:

filenameout = CommonDialog1.FileName

Text1.Text = filenameout

should be changed to:

filename2 = CommonDialog1.FileName

Text2.Text = filename2

This assumes the second file name text box is named “Text2”.

Then, change the line in the MakeNetlist_Click() subroutine from:

If filenameout <> "" Then

Unload Me

Call Main(filenameout, vbNullString)

. . . . . . . . .

to:

If (filenameout <> "" ) And (filename2 <> "") Then

Unload me

Call Main(filenameout, filename2)

. . . . . . . . . .

Converting an SDT ‘C’ Formatter to Visual Basic

1. Remove trailing semicolons - this can be done with search-and-replace.

2. Convert Options as described above.

3. Add Call to every function call with two or more arguments or remove parentheses from the argument string.. Example:

WriteString(0, “String to Write”);

becomes:

Call WriteString (0, “String to Write”)

or:

WriteString 0, “String to Write”

4. Remove AddSymbol() calls, replacing these with Basic string variable definitions. Remove the definitions of the associated integer variables. Under SDT there were User-defined symbols as well as Standard symbols, but there are now only Standard Symbols – User-defined symbols (which were always strings) are replaced by Basic strings. Remove SetSymbol() calls (except SetSymbol(ExitType..)) and replace these with assignments to Basic strings. The one remaining allowable use of SetSymbol() is described below.

5. If , when attempting to run a converted formatter you get a “Sub or Function not defined” message on a call to an SDT formatter function, it’s probably because the function no longer does anything useful or is not relevant to the new environment, and has been omitted. Comment out any such lines. If you believe the function is needed, contact OrCAD Technical Support.

Other changes will be necessary, such as the syntax of “If” constructs, function declarations, and block structure.

Available Netlist Formatter Functions:

General information:

When a file parameter is specified, it must be one of the following values:

0 - The Session Log

1 - First (or only) output file

2 - Second (optional) output file

3 - A temporary file, which is deleted when processing is complete

Note: When a formatter is being debugged outside of the Capture environment, all text sent to file 0 will be displayed in pop-up messages boxes instead.

All Basic strings passed as parameters to a DLL function should be defined long enough to hold the maximum length of data that may be returned the function. For safety, pad strings out to 255 bytes before calling a DLL function. When a string is modified by a function, it is returned in the format of a “C” string, that is, terminated by a 0 (null). However, the length of the string to Basic is still the padded length (255). The Basic len() function will not yield the correct string length. Therefore a CLen() function has been provided to compute the actual “C” string length. Before doing manipulations in Basic on strings returned by DLL functions, convert them to the proper Basic length by code such as the following:

Dim Reference As String

Reference = String (255, vbNullChar)

‘ sets long string length

CopySymbol (ReferenceString, Reference)

‘ returns a null-terminated string

Reference = Left (Reference, CLen(Reference))

‘ makes Basic length = C length

Function names are listed in Boldface Italics, Basic keywords in Boldface, user-supplied parameters in italics.

AccessKeyWord  (ByVal symbol As String) As Long

Uses the input symbol string containing a pipe symbol keyword to make current the next line in the Pipe Text containing that keyword. The standard symbols Keyword and Pipeline are loaded from the current line. The function returns 1 on success or 0 otherwise. If the function returns 0, then no action has been taken – the position in the pipe text is reset to be the same as before the call and the Keyword and Pipeline standard symbols retain their previous values. This function will return 0 when there is no pipe text in the netlist.

CLen (ByVal stringa As String) As Long

Returns the length of stringa considering it a C-style (null terminated string). Returns the number of characters up to but not including the first null character. Used to truncate a string in Basic to its null-terminated length.

ConcatFile  (ByVal filea As Long, ByVal fileb As Long) As Long

Places the contents of  the file specified by fileb at the end of the file specified by filea. Usually fileb will be 3, and filea will be 1 or 2, which will concatenate the temporary file to one of the regular output files. This is usually done after all writing to both files is complete. Neither filea nor fileb may be 0.

CopySymbol  (ByVal sourceId As Long, ByVal symbol As String)

Copies a string-type standard symbol, such as current part reference designator, or current net name,  identified by sourceid into the user string symbol . sourceid can be any of the values defined as string-type values (GetStandardSymbol() is used to obtain values of non-string [scalar] standard symbols) The valid source symbol id’s are listed in a separate section below. This function can be used with any standard symbol except:

TypeCode

PinIndex

PinType

NetNumber

PartIndex

Example:

Dim Myreference As String * 255

CopySymbol (ReferenceString, Myreference);

EndNode  () As Long

Returns 1 if there are no more pins on the current net.  This function is redundant, since NextNetPin (), typically used in a loop iterating over net pins, returns 0 when the net’s pins are exhausted.

ExceptionsFor  (ByVal stringa As String, ByVal symbol As String) As Long

Modifies stringa according to the format specified in symbol,  which must be one of “EDIF”, “ADF”, or “MOD”. The characters in stringa are changed or verified according to the particular format. For EDIF, words are substituted in stringa as follows: “MINUS” for “-“, “PLUS” for “+”, “BAR” for “/” or “\”.

For ADF,  if the leftmost part stringa matches “AND", "NAND", "OR", "NORF" , or "NOR", stringa is truncated to the length of the matching string .

For MOD, if the leftmost part of stringa matches "AND" ,  "NAND" , "OR" , "NOR", "XOR", “XNOR", "LAND",  "LNAND", "LOR", “LNOR", "LXOR", "LXNOR", "RAM",  or "ROM", stringa is truncated to the length of the matching string.

FirstNetPin  () As Long

Makes the first pin of the current net the current pin. Returns 1 if successful, or 0 if the net has no pins (which should not occur).

FirstPipe  () As Long

Makes the first line of pipe text (if any) the current line and stores it in the standard variable PipeLine. Returns 0 if there is no pipe text. If the line begins with “’|’ (single quote-vertical bar) the next line is read and the token after the next line’s vertical bar is made the current Pipe Keyword.

GetFirstNetProperty  (ByVal PropertName As String, ByVal propvalue As String) As Long

GetFirstPartProperty  (ByVal PropertyName As String, ByVal propvalue As String) As Long

GetFirstPinProperty  (ByVal PropertyName As String, ByVal propvalue As String) As Long

GetNextNetProperty  (ByVal PropertName As String, ByVal propvalue As String) As Long

GetNextPartProperty  (ByVal PropertName As String, ByVal propvalue As String) As Long

GetNextPinProperty  (ByVal PropertName As String, ByVal propvalue As String) As Long

You can access net, part, and pin properties in two ways: You can either iterate over all the properties for a given object, and process only the ones of interest, or you can ask for a specific property by name. The value may be empty, but the property may be present. Here’s an example of iteration over all properties for a Net:

DIM PropName as String * 255, PropValue as String * 255

DIM Result as Long

Result = GetFirstNetProperty(PropName, PropValue)

while Result = 1

PropName = Left(PropName, CLen(PropName))

PropValue = Left(PropValue, CLen(PropName))

if PropName = { property of interest }

{ process property }

End If

Result = GetNextNetProperty(PropName, PropValue)

Wend

GetNamedNetProperty  (ByVal ProperytName As String, ByVal propvalue As String) As Long

GetNamedPartProperty  (ByVal ProperytName As String, ByVal propvalue As String) As Long

GetNamedPinProperty  (ByVal ProperytName As String, ByVal propvalue As String) As Long

These functions query the current part, pin or net for the specified property. The functions return 1 if the property exists on the part, pin, or net.  The returned value may be empty, in which case propvalue will be returned as a 0-length string. If the property does not exist on the part, pin, or net, the function returns 0.

GetNetCount  () As Long

Returns the number of nets in the design.

GetStandardSymbol  (ByVal symbolId As Long) As Long

Returns the value of any of the “scalar” standard symbols (see Standard Symbols below for the definition and usage of each symbol). The defined scalar symbols are:

TypeCode

PinIndex

PinType

NetNumber

PartIndex

GetSymbolChar  (ByVal index As Long, ByVal symbol As String) As Long

Returns the character specified by index from the Basic string symbol. Index is zero-based (the first character is character 0) and must be less than the length of symbol.

InsertFileInPM  (ByVal FileName As String) As Long

Inserts the given Filename in the Outputs folder of the Project Manager window as a Report file. The file may then be selected and edited from the PM. See the WIRELIST sample project for an example of the use of this function.

IsKeyWord  () As Long

Returns 1 if the current pipe text line contains the current Keyword.

LoadFieldString  (ByVal RefDes As String) As Long

Makes the part whose reference designator matches RefDes the current part. Returns 1 on success, or returns 0 if the specified reference designator cannot be found.

LoadFirstPin  () As Long

Makes the first pin for the Current Part the Current Pin. Note that said pin may or may not be connected (i.e., may or may not belong to a net).

LoadInstance  () As Long

Makes the next part the Current Part. Returns 1 if successful. Returns 0 if there are no more parts. Parts will be retrieved in order by reference designator, unless there has been a call to SetAccessType() with an argument other than “REFERENCE”. In this case, the part table will be sorted on some other field. The first call to this function must be preceded by a call to RewindInstanceFile().

LoadPin  () As Long

Makes the next pin for the Current Part the Current Pin. Returns 0 if there are no more pins on the part, otherwise returns 1.

MakeNetCurrent  (ByVal nNet As Long)

Makes current the net whose number is given by nNet. Nets are numbered from 0. The number must between 0 and the total number of nets (the value returned by GetNetCount() ) less one.

NextAccessType  () As Long

Makes current the next part that contains an access field different from the current part’s access field. SetAccessType() should be called first to establish the field used for comparison. This function is typically used to get all the different part types in a design.

NextInstance  () As Long

This is a complement to NextAccessType - this make current the next part with

the same value for the specified access type. Use SetAccessType() first. This function may be used to accumulate all the parts of a given type.

NextNetPin  () As Long

Makes the next pin for the Current Net the Current Pin. Returns 0 if the net has no more pins, otherwise returns 1.

NextPipe  () As Long

Make the next line of pipe text, if any, current (loads it into the PipeLine standard variable). If the line begins with “’|’ (single quote-vertical bar) the next line is read and the token after the next line’s vertical bar is made the current Pipe Keyword.

PadSpaces  (ByVal symbol As String, ByVal count As Long) As Long

Sets the null-terminated length of symbol to count, either adding spaces to, or truncating symbol, depending upon whether count is greater than, or less than, the current length of symbol, respectively.

RewindInstanceFile ()

Initializes scanning of parts. Call LoadInstance() to access the first and successive parts.

SetAccessType  (ByVal stringa As String) As Long

Sets the current access type (as used  by NextAccessType()) according to stringa, which must be one of “PARTVALUE”, “LIBRARY”, “LOOKUP”, or “REFERENCE”. This function also sorts the part table by the specified field.

SetCharSet  (ByVal stringa As String) As Long

Sets the list of valid characters to be used by the SymbolInCharSet() function.

SetFirst  (ByVal symbol As String) As Long

Makes current the first part, pin, or net,  depending upon symbol. If symbol is “ALL”

The first part is made current, the first pin of that part is made current, and the net to which that pin is connected is made current. If symbol is “SIGNALS”, the first net, first pin of the net, and the part the pin is connected to, is made current. If symbols is “NODES”, the first pin for the current part is made current, as is the net to which that pin is connected.

SetIndexByRef  (ByVal symbol As String) As Long

Makes current the part whose reference designator is given by symbol. Returns 1 if successful. Returns 0 if the no part is found with a matching reference designator.

SetNext  (ByVal symbol As String) As Long

Makes current the next net, part, or pin depending upon symbol. If symbol is “ALL” the next part is made current, if any is made current, as is the part’s first pin and net connected to that pin (if any). If symbol is “SIGNALS” the next net is made current, as is the first pin for that net and the part to which the pin belongs. If symbol is “NODES”, the next pin for the current part is made current, as is the net to which that pin is connected, if any.

SetNumberWidth  (ByVal width As Long) As Long

Sets the width to which numeric values will be padded when written to a file by WriteInteger (). The value is initialized to 1.

SetPinMap  (ByVal index As Long, ByVal stringa As String)

The Pin Map is a list of 16 strings whose values can be written to a file using WriteMap(). The string indicated by index is set to the value of stringa. This is usually used to translate the values of the standard symbol PinType into meaningful strings. See PinType below. index must be between 0 and 15 inclusive.

SetSymbol  (ByVal symbolid as Integer, ByVal stringa As String)

This function is used to indicate the overall success of the netlisting process. If a warning or error condition is detected in netlist generation, a call to SetSymbol(ExitType, “W”) or SetSymbol(ExitType, “E”) should be made, respectively. At the end of netlist generation, a message box will pop up reporting an error or warning condition occurred. If SetSymbol() is called with an “E” as its second parameter, a subsequent call with a “W” will be ignored. The first parameter must be ExitType. All other values will be ignored.

You may also record your own error or warning conditions and report them however you choose. This function was provided for compatibility with older formatters.

SortByNumber  () As Long

Sorts the part table by reference designator. SetAccessType() also sorts the part table by different fields, depending upon its argument. When netlisting begins, the part table is sorted by reference designator, so this function is redundant, although it could be used to restore the part table to reference designator order after SetAccessType() was used to access the part table in a different order.

SortNets  ()

Sorts the net table alphabetically by signal (net) name. If  this is not done, the net table is left in the order the nets are extracted from the Capture database, which may not be related to any other ordering.

StripPath  (ByVal symbol As String) As Long

Removes the drive and directory path (if any) from symbol, retaining only the file name and extension. For example, if symbol contains “C:\TEMP\FILE.NET” before calling this function, it will contain “FILE.NET” after.

SymbolInCharSet  (ByVal symbol As String) As Long

Returns 1 if all characters in symbol are contained in the character set defined by SetCharSet(). Returns 0 otherwise. Must be preceded by a call to SetCharSet() to establish the valid character set.

SymbolLength  (ByVal symbolId As Long) As Long

Returns the length (C-style null-terminated) of the value of the standard symbol specified by symbolid.

WriteCrLf  (ByVal file As Long)

Writes a carriage return and line-feed to the specified file. Carriage returns are not automatically added to text sent to any file. Text is not written immediately to the Session Log (file 0), rather, it is accumulated in a buffer and written only when WriteCrLf(0) is called.

WriteInteger  (ByVal file As Long, ByVal theInt As Long) As Long

Writes the value of theint to the specified file padded to the current number width (as set by SetNumberWidth()).

WriteMap  (ByVal file As Long, ByVal index As Long) As Long

Writes the Pin Map string specified by  index to the file specified by file. All index values referenced should have been previously assigned string values by SetPinMap() calls, and must be between 0 and 15 inclusive.

WriteString  (ByVal file As Long, ByVal stringa As String)

Writes stringa to the specified file. The string can be a literal (in quotes) or a Basic string variable. If a Basic string variable is given, all characters up to, but not including, the first null character will be written. Text is not written immediately to the Session Log (file 0), rather, it is accumulated in a buffer and written only when WriteCrLf(0) is called.

WriteSymbol  (ByVal file As Long, ByVal symbolId As Long)

Writes the value of the specified string-type standard symbol to the file indicated. Text is not written immediately to the Session Log (file 0), rather, it is accumulated in a buffer and written only when WriteCrLf(0) is called.

Standard Symbols:

The following symbols are used in calls to CopySymbol() (for string-type symbols) and GetStdSymbol() (for scalar-type symbols). Calling the wrong function for a symbol type (i.e. CopySymbol() for a scalar) will yield an undefined result. Symbols are string types unless indicated.

The following standard symbols return values for the Current Net:

SignalNameString - The name of the current net.

TypeCode (scalar) - Returns the  type of net, S for power, P for Port, or L for all others. This can be checked by code such as :

I = GetStandardSymbol (TypeCode)

If I = Asc("S") Then

rem Net is power type

End If

NetNumber (scalar) - the sequence number of the current net starting from 0. When the netlist data is first loaded, nets are loaded in the net table in the order they are extracted from the Capture database.

The following standard symbols return values for the Current Part:

PartIndex (scalar) - the sequence number of the part within the part table starting from 0. Sequence numbers will be different before and after SortByNumber() or SetAccessType() are  called to sort the part table.

PartName - the value resulting from the Part Value Combine String on the Netlist options dialog

ModuleName - the value resulting from the PCB Footprint Combine String on the Netlist options dialog

ReferenceString - reference designator

TimeStamp - an 8-character hexadecimal number assigned to a part, guaranteed to be unique from that of any other part within the design.

FieldString1

FieldString2

FieldString3

FieldString4

FieldString5

FieldString6

FieldString7

FieldString8  - these 8 symbols return the 8 properties with names of “1ST PART FIELD”, “2ND PART FIELD”, “3RD PART FIELD”, and  “4TH PART FIELD”, etc, and correspond to the 8 field strings on parts in designs translated from SDT.

LibraryNameString - the name of the library from which the part came.

LookupNameString - the value used to refer to the part in the library

The following symbols return values for the Current Pin:

PinNumberString - the pin number of the current pin (may be alphanumeric)

PinNameString - the name of the current pin

PinIndex (scalar)  - the relative sequence number of the pin on the part (not implemented in Alpha release)

PinType (scalar) - the following values are defined:

0  -- Input

1 --  Bi-Directional

2 --  Output

3 -- Open Collector

4 -- Passive

5 -- Hi-Z

6 -- Open Emitter

7 -- Power

The following standard symbols return values for Pipe Text:

KeyWord - the current pipe text keyword

PipeLine - the current pipe text line

The following symbols call out fields on the Title Block of the main schematic for the design:

DocumentNumber

DateString

Revision

TitleString

Organization

AddressLine1

AddressLine2

AddressLine3

AddressLine4

FileName